Student ID: 001158836

Programme of Study: MSc Electrical and Electronic Engineering

| POWER ELECTRONICS LOGBOOK Dr. Yehdego Habtay | DISHA HARWALKAR ELEE-1165- Power Electronic Converters |
| --- | --- |

Table of Contents

1	BUCK CONVERTER LABORATORY	3

1.1	INTRODUCTION	3

1.2	ANALYTICAL COMPUTATION	3

a.	Calculating the Values of the Inductor and Capacitor	3

b.	Determining the Rated Values For the MOSFET, Diode and Capacitor	4

c.	Analytically Determining the Effects of the Non-Ideal Characteristics on Voltage Conversion Ratio and Efficiency of Buck Converter	4

d.	Calculating the Values of the Voltage Conversion Ratio and Efficiency	5

e.	Sketch of Waveforms for Each Component	6

f.	Critical Value of Load Resistance at Boundary of CCM and DCM	6

g.	Critical Value of Frequency at Boundary of CCM and DCM	7

1.3	HARDWARE AND SOFTWARE REQUIREMENTS	7

1.4	PROCEDURE	7

1.5	DISCUSSION	14

1.6	CONCLUSION	15

2	FLYBACK CONVERTER LABORATORY	16

2.1	INTRODUCTION	16

2.2	ANALYTICAL COMPUTATIONS	16

2.3	PROCEDURE	17

2.4	DISCUSSION	20

2.5	CONCLUSION	20

3	CLOSED LOOP PWM CONTROLLER LABORATORY	22

3.1	INTRODUCTION	22

3.2	PROCEDURE	22

3.3	DISCUSSION	26

3.4	CONCLUSION	26

TABLE OF FIGURES

Figure 1 A buck converter with measuring devices	3

Figure 2 Sketch of expected waveforms	6

Figure 3 Inductor Current Waveform	7

Figure 4 Inductor Voltage Waveform	8

Figure 5 Capacitor Current Waveform	8

Figure 6 Capacitor Voltage Waveform	8

Figure 7 Load current waveform	9

Figure 8 Switch current waveform.	9

Figure 9 Switch voltage waveform	9

Figure 10 Diode current waveform	10

Figure 11 Diode voltage waveform	10

Figure 12 Input power waveform	10

Figure 13 Output power waveform	11

Figure 14 CCM and DCM boundary by load variation	12

Figure 15 CCM and DCM boundary by frequency variation	12

Figure 16 Buck Converter Designed in Proteus	13

Figure 17 Diode current and voltage waveforms	13

Figure 18 MOSFET Current and Voltage waveforms	13

Figure 19 Inductor current and Voltage waveforms	14

Figure 20 Load current and voltage waveforms	14

Figure 21 A flyback converter with measuring devices	16

Figure 22 Load current	17

Figure 23 Switch current	17

Figure 24 Switch voltage	17

Figure 25 Diode current	18

Figure 26 Diode voltage	18

Figure 27 Capacitor current	18

Figure 28 Capacitor voltage	19

Figure 29 Transformer primary voltage	19

Figure 30 Transformer secondary voltage	19

Figure 31 Buck Boost converter with closed loop control	21

Figure 32 PWM Signal	21

Figure 33 Output current	21

Figure 34 Output Voltage	22

Figure 35 Inductor Current	22

Figure 36 Output voltage	25

LIST OF TABLES

Table 1 Calculated and Measured Conversion Ratio and Efficiency	11

Table 2 Duty cycle comparison	11

Table 3 Frequency effects comparison	12

Table 4 Inductor comparison	12

Table 5 Calculated vs simulated values.	19

Table 6 Effects of Varying the Duty Cycle	20

Table 7 Effects of Varying the Frequency	20

Table 8 Measured vs Calculated Values.	23

# BUCK CONVERTER LABORATORY

## INTRODUCTION

A buck converter is a non-isolated dc to dc converter widely used in industry to convert a higher input voltage to a lower output voltage efficiently. The converter consists of an active switch - usually a MOSFET- controlled by a pulse generator, a rectifier, and filter elements as shown in Figure 1. The buck converter operates in two modes which are the Continuous Conduction Mode (CCM) and Discontinuous Conduction Mode (DCM). This experiment will study the components, control mechanism and filtering in a typical buck converter.

Figure 1 A buck converter with measuring devices

Figure 1 shows the circuit that was constructed in MATLAB Simulink. The next subtopic will contain the parameters and manual computation used to aid understanding of the operation of a buck converter without simulation.

## ANALYTICAL COMPUTATION

In this subtopic, the values of certain parameters were evaluated using manual computations. The typical buck converter which had the following parameters was studied. The buck converter had an input voltage of 15V and was required to supply a 10 Ohms load with 10V and 1A. The pulse generator had a switching frequency of 125KHz. The inductor current ripple and output voltage peak to peak ripples were expected not to exceed 20% and 1 % respectively.

### Calculating the Values of the Inductor and Capacitor

To determine the value of the inductor and capacitor to be used, the value of the duty cycle D was calculated,

D=vgv=1015=0.667

The values of the permitted inductor current ripple and output voltage peak to peak ripples were also calculated,

∆IL=20100×1=0.2A

∆VC=1100×10=0.1V

The value of the inductor and capacitor were then determined as follows,

L=Vg-VDTs∆iL

L=(15-10)×0.667125×103×0.2

L=133.4μH

and

C=∆iLFSW×8×∆V

C=0.2125×103×8×0.1

C=2μF

### Determining the Rated Values For the MOSFET, Diode and Capacitor

Determining the rated values for the components requires deducing a component that can be used for the application while justifying the selection. The components determined were the MOSFET, Diode and Capacitor.

#### MOSFET

The minimum voltage that the MOSFET will be required to block during OFF state is 15V. Therefore, the MOSFET should be rated above 15V. The minimum current the MOSFET will be subjected to is 1A. Therefore, a 20V and 2A rated MOSFET would be optimal.

#### Diode

The minimum voltage that the Diode will be required to block during the ON state is 15V. A diode capable of blocking voltage above 15V should be selected. It would be subjected to a current of about 1A. Therefore, a 20V and 2A rated diode should be selected for this operation.

#### Capacitor

The minimum voltage that the Capacitor will be required to withstand will be an output voltage of 10V. The selected capacitor should be rated above 10V. The current the capacitor would be subjected to will range from 0 to about 1A. Therefore, selecting a 15V and 2A rated capacitor would be optimal.

### Analytically Determining the Effects of the Non-Ideal Characteristics on Voltage Conversion Ratio and Efficiency of Buck Converter

There are factors that affect the voltage conversion ratio and efficiency of a buck converter. These factors are the ON resistance of the MOSFET RDSON, the diode voltage drop VDON, the inductor winding resistance RL, and the Equivalent Series Resistance (ESR) of Capacitor.

To derive the equation of the voltage conversion ratio and the efficiency which are affected by non-ideal characteristics, the capacitor ESR was not considered because it does not significantly affect the voltage drop in the buck converter. The equations are derived below,

During ON time

VLON=Vg-IORDSON-IORL-VO … (1)

During OFF time

VLOFF=-VO-VDON-IORL … (2)

Equation (1) and (2) were substituted into the Volt Second Balance equation

VLONDT+VLOFF1-DT=0

VgD-IORDSON+VDOND-VO-VDSON-VDOND-IORL=0

D=VO+IORL+VDONVg-IORDSON+VDON

To find the Efficiency of the buck converter,

η=POPIN+Plosses×100%

The formula for the power loss is given below.

Plosses=Pcond+Pswloss

The formulas for the switching and conduction losses are given below,

Pswitch=12VgIO(tswoff+tswon)fsw

Pconduction=I2ORDSON×D+0.7IO1-D+I2ORL

### Calculating the Values of the Voltage Conversion Ratio and Efficiency

The values of the voltage conversion ratio and the efficiency were calculated as shown below,

D=10+0.064+0.6815-0.016+0.68

D= 0.686

Pswloss=12×15×1×278+48×10-9×125×103

(the rise time and fall time were gotten from the MOSFET datasheet and is multiplied by 2 to get the on time and off time)

Pswloss=0.236W

The value of inductor resistance = 0.064 ohms used is gotten from the ATCA-05-141M datasheet which is a 140uH inductor. The site used was mouser.co.uk

The diode forward voltage VDON = 0.68 V and MOSFET RDSON = 0.016 Ohms were gotten from the datasheets of MUR405G and IRFIZ48NPBF, respectively.

The conduction loss is given below,

Pconduction=I2ORDSON×D+0.7IO1-D+I2ORL

Pcond=12×0.016×0.68+0.711-0.68+12×0.064

Pcond=0.302W

Summing up the conduction and switching losses,

Plosses=0.236+0.302

Plosses=0.538W

The efficiency is thus calculated,

η=IOVOIOVO+Plosses×100

η=1010+0.538×100

η=94.9%

### Sketch of Waveforms for Each Component

The expected waveforms for the components were sketched in Figure 2.

Figure 2 Sketch of expected waveforms

### Critical Value of Load Resistance at Boundary of CCM and DCM

The critical value for the load resistance at the boundary of the Continuous Conduction Mode and Discontinuous Conduction Mode was calculated as follows,

IL=VOR=DVgR

Find IL minimum

IL-∆IL2=DVgR-Vg-VODT2L

Equate the above equation to zero.

DVgR-Vg-VODT2L=0

R=2Lfsw1-D

R=2×133.4µ×125×1031-0.667

R=100.15Ω

### Critical Value of Frequency at Boundary of CCM and DCM

The critical value for the frequency at the boundary of the Continuous Conduction Mode and Discontinuous Conduction Mode was calculated as follows,

fsw=R(1-D)2L

fsw=10(1-0.667)2×133.4µ

fsw=12.48KHz

## HARDWARE AND SOFTWARE REQUIREMENTS

The hardware used for this experiment was a University of Greenwich desktop computer at the Nelson building on Medway campus which has a core i5 8th generation processor. The software used for this lab were MATLAB Simulink© and Proteus.

## PROCEDURE

The non-ideal characteristics for the MOSFET and diode were included in the designed circuit as shown in Figure 1. The inductor resistance RL was added to the inductor as a resistor in series with it. The waveforms of each components over 3 switching intervals are shown below.

- Inductor

Figure 3 Inductor Current Waveform

The inductor ripple measured here is 0.2091A

Figure 4 Inductor Voltage Waveform

- Capacitor

The waveform for the capacitor current and voltages over three switching cycles are given below.

Figure 5 Capacitor Current Waveform

Figure 6 Capacitor Voltage Waveform

The measured capacitor voltage ripple is 0.07187V

- Load

The waveform for the load voltage is a duplicate of the capacitor. The waveform for the load current is shown below.

Figure 7 Load current waveform

- Switch (MOSFET)

Figure 8 Switch current waveform.

Figure 9 Switch voltage waveform

- Diode

Figure 10 Diode current waveform

Figure 11 Diode voltage waveform

- Power

A power measurement block was used to measure the input and output power. Their waveforms are shown.

Figure 12 Input power waveform

The value for the input power was measured as 9.806W.

Figure 13 Output power waveform

The value for the output power is measured as 9.412W

- Voltage Conversion Ratio and Efficiency

The calculated and measured conversion ratios and efficiencies are shown below.

Table 1 Calculated and Measured Conversion Ratio and Efficiency

|  | Calculated | Measured | Error (%) |
| --- | --- | --- | --- |
| Conversion Ratio | 0.686 | 0.648 | 5.5 |
| Efficiency | 94.9 % | 95.9 % | 1 |

Investigating Impacts of Changing the Duty Cycle, Load, Switching Frequency, and Inductance.

- Duty Cycle

The duty cycles were adjusted between these values 33.33%, 66.67% and 83.34% and their effects were compared in the table below.

Table 2 Duty cycle comparison

|  | 33.33% Duty Cycle | 66.67% Duty Cycle [Normal] | 83.34% Duty Cycle |
| --- | --- | --- | --- |
| Output voltage, VO | 4.489 V | 9.722 V | 12.31 V |
| Peak-to-peak Inductor Current Ripple, ∆IL | 0.2095 A | 0.2091 A | 0.1304 A |
| Peak-to-peak Capacitor Voltage Ripple, ∆v | 0.072 V | 0.07187 V | 0.03668 V |
| Output Power | 2.038 W | 9.412 W | 15.11 W |

- Load

The instructions were to reduce the load but reducing the load increased the current rather than bringing it closer to the boundary of the Continuous Conduction Mode (CCM) and Discontinuous Conduction Mode (DCM). Therefore, the load was increased.

The CCM and DCM boundary was reached at 110 Ohms and the inductor current waveform is shown.

Figure 14 CCM and DCM boundary by load variation

- Frequency

The frequency was reduced until the inductor current reached the boundary between CCM and DCM which was about 17.1KHz as shown below.

Figure 15 CCM and DCM boundary by frequency variation

The frequency of the pulse generator was changed to 250KHz. Some parameters were compared with the original as shown in the table below.

Table 3 Frequency effects comparison

|  | 125 KHz | 250 KHz |
| --- | --- | --- |
| Output voltage, VO | 9.722 V | 9.707 V |
| Peak-to-peak Inductor Current Ripple, ∆IL | 0.2091 A | 0.1039 A |
| Peak-to-peak Capacitor Voltage Ripple, ∆v | 0.07187 V | 0.01727 |
| Output Power | 9.421 W | 9.412 W |

- Inductor

The values of the inductor were modified and the effect on the inductor current, inductor current ripple and output voltage ripple are shown in the table below.

Table 4 Inductor comparison

|  | 50 uH | 133.4 uH | 400 uH |
| --- | --- | --- | --- |
| Average Inductor Current, IL | 0.6889 A | 0.8654 A | 0.9353 A |
| Peak-to-peak Inductor Current Ripple, ∆IL | 0.5609 A | 0.2091 A | 0.0694 A |
| Peak-to-peak Capacitor Voltage Ripple, ∆v | 0.1934 V | 0.07187 V | 0.0238 V |

- Proteus

The Proteus application was used to simulate the buck converter circuit as shown below.

Figure 16 Buck Converter Designed in Proteus

The waveforms are given below.

Diode

Figure 17 Diode current and voltage waveforms

MOSFET

Figure 18 MOSFET Current and Voltage waveforms

Figure 19 Inductor current and Voltage waveforms

Inductor

Load

Figure 20 Load current and voltage waveforms

The input power was measured as 9.3W, output power was 5.46W. Voltage conversion ratio was measured as 0.492 while efficiency was 58.7%

## DISCUSSION

The measured inductor current ripple in Figure 3 matches the analytical results calculated as the measured current ripple (0.2091 A) was similar to the calculated inductor current ripple which had a value of 0.2 A. The measured capacitor voltage ripple in Figure 6 had a value of 0.07187 V which was similar to the calculated value of 0.1 V. The inductor voltage waveform in Figure 4 was expected because during the ON time the inductor voltage rises to Vg-V = 15-10 = 5 V and during the OFF time the inductor voltage reduces to -V = -15 V as shown in Figure 4.

The duty cycle used with the switch (or MOSFET) is responsible for the switch and diode voltage waveforms blocking voltage for the OFF and ON times respectively in Figures 9 and 11. The inductor current in Figure 3 is the combination of the switch and diode currents in Figure 8 and 10 because the switch charges the inductor during ON time and the inductor discharges into the diode during OFF time. Meanwhile, the inductor and the capacitor supply the load during the OFF time as shown in the current waveforms of the inductor, capacitor, and load current in Figures 3, 5 and 7.

The input and output power in Figures 12 and 13 were slightly under 10 W due to non-ideal characteristics added and internal losses in the components. The calculated and measured values in Table 1 are very similar. The efficiency was not 100% due to the losses added.

The duty cycle was varied as shown in Table 2 and it shows that an increase in duty cycle of a buck converter would result in an increase in the output voltage. The voltage and current ripples increased during a decrease in the duty cycle but was not significant. The output power also increased with an increase in the duty cycle. This means the magnitude of the voltage output can be controlled by varying the duty cycle.

The inductor reached the boundary of CCM and DCM in Figure 14 when the load was increased to 110 Ohms. The calculated value was 100.15 Ohms. A nearly 10 Ohms difference was observed. The inductor reached the boundary of CCM and DCM in Figure 15 when the frequency was set to 17.1Khz. The calculated value was 12.48 KHz. Therefore, exceeding the limits should be avoided.

The frequency of 125KHz was increased to 250KHz in Table 3 and the output voltage reduced but not significantly, the ripples in the inductor current and capacitor current reduced and the output power reduced. This means the switching frequency can be increased to reduce the size of the filtering elements.

The inductor was varied as shown in Table 4. Increasing the inductor means increasing the average inductor current. Increasing the inductor value also decreased the amount of ripples in the inductor current and output voltage.

The buck converter designed on Proteus were not adjusted (given non ideal characteristics) because the components already had non idealities which dropped the output voltage heavily to about 7V. Therefore, the efficiency was very low at 58.7%

## CONCLUSION

A buck converter is used to efficiently convert a higher input voltage to a lower output voltage. In this experiment, a buck converter was modelled and simulated on Simulink and Proteus application. The expected parameters were calculated before the experiment and the waveforms were extracted. The simulated and calculated models were similar in values. The effects of varying parameters such as the duty cycle, inductor values and switching frequency were studied. It was found that increasing the duty cycle increases the output voltage, increasing the switching frequency reduces ripples and increasing the inductor value reduces ripples. It can be concluded that in applications where space is important increasing the frequency might be more suitable than increasing the size of inductor while the opposite is applicable for applications where space is not important.

# FLYBACK CONVERTER LABORATORY

## INTRODUCTION

Flyback converters are isolated DC to DC buck-boost converters that use a coupled inductor as opposed to an isolation AC transformer as shown in figure 21. It can operate in two modes which are the Continuous Conduction Mode (CCM) and the Discontinuous Conduction Mode (DCM).

Figure 21 A flyback converter with measuring devices

The flyback converter stores energy during the ON state and transfers it to the load during the OFF state. In theory, the voltage gain of the flyback is infinite. Since it does not transfer energy directly, the flyback converter is mostly used in low power applications (typically below 100W). Figure 21 shows the design of a flyback converter using the MATLAB Simulink software by MathWorks®.

## ANALYTICAL COMPUTATIONS

The magnetizing inductor current ripple ∆iLM, capacitor voltage ripple, switching voltage and diode voltage during ON time have been calculated below,

∆iLM=VgLM×DT

∆iLM=150×0.343100

∆iLM=0.5145 A,

∆VC=-VORCDT

∆VC=-15×0.343200

∆VC=-0.02575 V,

VSW=Vg+VON1N2

VSW=150+1515030=225 V,

VD=-VO-VgN2N1

VD=-15-15030150=-45 V

## PROCEDURE

For this simulation, the values inputted into the linear transformer were converted to RMS values where necessary by dividing it by √2. The relevant waveforms for the inductor, switch (MOSFET), load, diode, capacitor, and transformer are shown below.

- Load

The current waveform for the load is coloured yellow while the voltage waveform is coloured blue, and they are shown below.

Figure 22 Load current

- Switch

Figure 23 Switch current

Figure 24 Switch voltage

The measured value for the MOSFET voltage is 224.9 V.

- Diode

Figure 25 Diode current

Figure 26 Diode voltage

The measured value for the diode voltage is -44.10 V.

- Inductor

The inductor waveforms could not be included because the magnetizing inductor LM is located inside the transformer in Simulink.

- Capacitor

Figure 27 Capacitor current

Figure 28 Capacitor voltage

The value for the capacitor voltage ripple is 0.02613.

- Transformer

The input and output voltage waveforms for the transformer are shown below.

Figure 29 Transformer primary voltage

Figure 30 Transformer secondary voltage

- Theoretical calculations vs Simulation

The table below compares the values of the theoretical calculations with simulated values.

Table 5 Calculated vs simulated values.

|  | Calculated | Simulated | Error (%) |
| --- | --- | --- | --- |
| Peak-to-peak Capacitor Voltage Ripple, ∆v | 0.025725 V | 0.02614 V | 1.6 |
| Switching Frequency | 225 V | 224.9 V | 0.044 |
| Diode voltage | -45 V | -44.10 V | 2 |

- Studying the Effects of Varying the Duty Cycle

The duty cycle was varied, and the measured results are shown in the table below.

Table 6 Effects of Varying the Duty Cycle

|  | 20% | 34.3% [Normal] | 60% |
| --- | --- | --- | --- |
| Peak-to-peak Capacitor Voltage Ripple, ∆v | 0.009784 V | 0.02613 V | 0.1211 V |
| Output voltage, VO | 7.24 V | 14.99 V | 41.13 V |
| Output current, IO | 0.724 A | 1.499 A | 4.113 A |

- Studying the Effects of Varying the Frequency

The frequency was varied, and the measured results are shown in the table below.

Table 7 Effects of Varying the Frequency

|  | 50 KHz | 100 KHz [Normal] | 200 KHz |
| --- | --- | --- | --- |
| Peak-to-peak Capacitor Voltage Ripple, ∆v | 0.05499 V | 0.02613 V | 0.01274 V |
| Output voltage, VO | 15.9 | 14.99 V | 14.57 V |
| Output current, IO | 1.59 | 1.499 A | 1.457 A |

## DISCUSSION

In Figures 22-30, the flyback converter possesses the regular waveforms of a buck-boost converter. It was not possible to obtain the waveform for the magnetizing inductor since it is located inside the coupled inductor (transformer). The waveforms for the primary and secondary voltages for the transformer were obtained in Figure 29 and 30. It shows the winding voltage of 150 V and 30 V, respectively. The calculated and simulated values were very similar in magnitude as shown in Table 5. The small discrepancies are due to the internal losses in the circuit.

The duty cycle was varied as shown in Table 6. Increasing the duty cycle increased the capacitor voltage ripples, the output voltage and the output current. Therefore, the duty cycle can be used to control the magnitude of the output, although, the ripples would increase which means more filtering would be needed.

Increasing the switching frequency reduces the capacitor voltage ripples, output voltage and output current as shown in Table 7. Increasing the switching frequency could replace the use of large filter elements and is ideal in low power application or applications in which space is important.

## CONCLUSION

Flyback converters are isolated converters that use coupled inductors instead of isolation transformers. Isolation could help in safety applications, reducing transformer size, minimizing current stresses, and creating more outputs. In this experiment, Simulink was used to study a flyback converter. It was found to behave like a regular buck boost converter. An increase in the duty cycle led to an increase in capacitor voltage ripples, output voltage and output current. While increasing the frequency led to a decrease in voltage ripples, output voltage and output current.

# CLOSED LOOP PWM CONTROLLER LABORATORY

## INTRODUCTION

In this experiment, a boost converter which is controlled by a closed loop PWM controller was modelled and simulated using Proteus. The purpose of a closed loop controller is to maintain a constant output under load variations and input variations. The Closed Loop PWM controlled boost converter is shown below.

Figure 31 Buck Boost converter with closed loop control

## PROCEDURE

The waveforms obtained from the simulation are shown below:

Figure 32 PWM Signal

Figure 33 Output current

Figure 34 Output Voltage

Figure 35 Inductor Current

- Comparing Measured and Calculated Values

When R1 is 28KΩ,

The measured value for the period of the PWM is 173uS. The duty cycle of the PWM is 0.064. The output voltage ripple is 0.8V. The output current ripple is 3mA and the inductor current ripple is 2.13A

Period, T of the PWM signal from the graph is 359u – 186u = 173u, frequency, F = 5.78KHz and the TON is 197u – 186u = 11u. Duty cycle is TON/T = 11u/ 173u = 0.0636 or 6.36%

While the calculated values are,

From the datasheet of UC1845N

fosc=1.72RRT×CCT

fosc=1.7228k×5n

fosc=12.29KHz

Switching frequency is half of oscillator frequency = 12.29 / 2 = 6.145KHz

Period, T is = 162.7u

Inductor current ripple,∆iL=VgLDT

∆iL=20×0.065×173u110u=2A

Output voltage ripple, ∆VO=VO×DTRC

∆VO=25×0.065×173u245×50u=0.0229V

A summary of the measured against the calculated values are shown in the table below,

Table 8 Measured vs Calculated Values.

|  | Measured Values | Calculated Values | Error(%) |
| --- | --- | --- | --- |
| Switching frequency | 5.78KHz | 6.145KHz | 6.31 |
| Period, T | 173u | 162.7u | 5.9 |
| Inductor current ripple,∆iL | 2.13A | 2A | 6.1 |
| Output voltage ripple, ∆VO | 0.8V | 0.0229V | 97 |

- Investigating the Variation of R1

When R1 is 50KΩ,

Period, T of the PWM signal from the graph is 631u – 325u = 306u and the TON is 336u – 325u = 11u. Duty cycle is TON/T = 11u/ 306u = 0.0359 or 3.59%

While the calculated value is,

From the datasheet of UC1845N

fosc=1.72RRT×CCT

fosc=1.7250k×5n

fosc=6.88KHz

Switching frequency is half of oscillator frequency = 6.88 / 2 = 3.44KHz

Period, T is = 290.698uS.

The output voltage ripple is 0.8V. The output current ripple is 3mA and the inductor current ripple is 2.139A

When R1 is 10KΩ,

Period, T of the PWM signal from the graph is 387u – 324u = 63u and the TON is 335u – 324u = 11u. Duty cycle is TON/T = 11u/ 63u = 0.175 or 17.5%

While the calculated value is,

From the datasheet of UC1845N

fosc=1.72RRT×CCT

fosc=1.7210k×5n

fosc=34.4KHz

Switching frequency is half of oscillator frequency = 34.4 / 2 = 17.2KHz

Period, T is = 58.14uS.

The switching frequency changes. When R1 is higher the switching frequency reduces but when R1 is lower the switching frequency increases.

The output voltage ripple is 0.7V. The output current ripple is 3mA and the inductor current ripple is 2.14A

- Increasing Relevant Components

The components that need to be adjusted for the output voltage to increase are the resistors forming the voltage divider. From the datasheet of UC1845N the formula for calculating the resistors to be changed was gotten and is given below,

VOFFSET=RCSFRCSF+RP×VREF

VOFFSET is 2.5V and was acquired from the datasheet of UC1845N.

Since a 5% increase is required to the output voltage VREF is calculated to be,

VREF=5%×VO+VO (VO is 25V and was measured as the output)

VREF=0.05×25+25

VREF=26.25V

RCSF was selected to be 3kΩ, the value of RP needs to be determined,

RP=RCSF×VREFVOFFSET-RCSF

RP=3k×26.252.5-3k

RP=28.5kΩ

The values of 3kΩ and 28.5 kΩ were used in the voltage divider and the output voltage was observed to be 26.25V as shown in the graph below,

Figure 36 Output voltage

## DISCUSSION

The figures 32-35 show the normal operation of a boost converter during switching cycles. The calculated values were lower than the measured values of the period of the PWM signal as shown in Table 8. This is due to non-ideal characteristics inherent in the circuit. This was also the case for the switching frequency and the inductor ripple current. Although, the calculated and measured values of the output voltage ripple were far in valuation. This is due to the components in Proteus spiking the ripple.

It was observed that increasing the magnitude of the resistor R1 connected to the VREF and RT/CT would mean reducing the switching frequency of the PWM and vice versa. Although, the output voltage, ripple current and voltage remained the same.

Controlling the output voltage is performed using the voltage divider providing a feedback to the VREF pin from the output voltage. The reference voltage and resistors for the voltage dividers need to be determined then feedback is sent to the integrated circuit which uses a comparator circuit to correct the signal to the MOSFET (switch).

## CONCLUSION

The purpose of a closed loop PWM controller is to maintain a constant output under load and input variations. A boost converter was modelled using Proteus and the switching frequency was controlled using the resistor R1 connected to the VREF and RT/CT. The magnitude of the load was controlled by adjusting the resistors which form a voltage divider at the output and sends feedback to a comparator.